NEXGO Android Smart PinPAD

Programmer's Manual

Host Command Protocol Specification

Revision 1.2 - 2026-07-03

This document specifies the host command protocol implemented by the NEXGO Android Smart PinPAD application. The device performs local L2 card interaction, secure PIN entry, prompts, key management, and card-reader control. Host authorization and settlement remain outside the device.

# Section 1    Communication Protocol

All command packets are transmitted through the selected serial transport. The application supports RS232 and USB CDC virtual serial operation. The transport layer validates start and end controls, LRC, partial-frame timeout, ACK/NAK retry, and session termination.

Control characters:

| Packet control values |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | 02 hexadecimal. Starts a transaction frame. |
| <ETX> | 1 | 03 hexadecimal. Ends a transaction frame. |
| <SI> | 1 | 0F hexadecimal. Starts an administration frame. |
| <SO> | 1 | 0E hexadecimal. Ends an administration frame. |
| <ACK> | 1 | 06 hexadecimal. Positive acknowledgement. |
| <NAK> | 1 | 15 hexadecimal. Transmission error acknowledgement. |
| <EOT> | 1 | 04 hexadecimal. Session termination, format error, or cancellation. |
| <FS> | 1 | 1C hexadecimal. Field separator. |
| <SUB> | 1 | 1A hexadecimal. Subfield delimiter. |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Send request frame. | -> |  |
|  | <- | Send <ACK> after a valid frame is received and accepted for processing. Send <NAK> on transmission error. Send <EOT> on packet format error where specified. |
|  | <- | Send command response frame when processing completes. |
| Send <ACK> if response is valid, <NAK> if response has LRC error, or <EOT> to cancel. | -> |  |
|  | <- | Administration responses send final <EOT> after host <ACK>, except commands 11 and 14. Transaction responses do not send a final <EOT>. |

# Section 2    Administration and Maintenance Messages

Message 02    Load Master Key or Working Key

Format:        <SI>02[Data]<SO>[LRC] (request frame)

<SI>02[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Loads a clear key into the selected secure key slot or working-key role.

Implementation:  K0 writes the device master-key slot. P0, M3, and D0 are stored as working PIN, MAC, and data keys. P0 uses a random wrapper master key internally.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 02 | 2 | Message ID |
| [Key ID] | 1 | '0' to '9' and 'A' to 'G'. |
| [Key value] | 16, 32, or 48 | Hexadecimal key value. Single, double, and triple length keys are accepted according to the selected algorithm. |
| <FS> | 1 | Optional field separator. Present when key attributes follow. |
| [Usage] | 2 | Optional key usage. K0 key encryption, K1 key transport, P0 PIN encryption, M3 MAC, D0 data encryption. |
| [Mode] | 1 | Optional key mode. Common values include D, E, G, V. |
| [Algorithm] | 1 | Optional algorithm. Default is T when omitted. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 02 | 2 | Message ID |
| 02 | 2 | Message ID echoed with the original payload on success. |
| ? | 1 | Present only in the error frame. |
| [Error code] | 1 | Command-specific key loading error code. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 02 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 02. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 04    Check Master Key

Format:        <SI>04[Data]<SO>[LRC] (request frame)

<SI>04[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Queries whether a key exists and optionally returns the stored key usage metadata.

Implementation:  If secure key material exists but local metadata was removed by reinstall, PIN commands fail safely and require key injection again.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 04 | 2 | Message ID |
| [Key ID] | 1 | Key slot to query. |
| [Attribute flag] | 1 | Optional. '1' requests key attribute information. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 04 | 2 | Message ID |
| [Status] | 1 | 'F' key exists, '0' key does not exist. |
| [Usage] | 2 | Returned only when requested and available. |
| <FS> | 1 | Field separator. |
| [Mode] | 1 | Returned key mode. |
| [Algorithm] | 1 | Returned algorithm. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 04 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 04. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 05    Load Serial Number

Format:        <SI>05[Data]<SO>[LRC] (request frame)

<SI>05[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Compatibility command. The hardware serial number is not changed.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 05 | 2 | Message ID |
| [Serial number] | 0 to 16 | Compatibility serial number value. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 05 | 2 | Message ID |
| [Serial number] | 0 to 16 | Echo of accepted serial substring. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 05 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 05. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 06    Get Serial Number

Format:        <SI>06[Data]<SO>[LRC] (request frame)

<SI>06[Data]<SO>[LRC] (response frame)

Message length:  Fixed request.

Usage:           Returns the device serial number.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 06 | 2 | Message ID |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 06 | 2 | Message ID |
| [Serial number] | Variable | Device serial number from Android or NEXGO SDK. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 06 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 06. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 08    Select Active Master Key

Format:        <SI>08[Data]<SO>[LRC] (request frame)

<SI>08[Data]<SO>[LRC] (response frame)

Message length:  Fixed.

Usage:           Selects the active key used by MK/SK PIN and MAC commands.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 08 | 2 | Message ID |
| [Key ID] | 1 | Key id to select. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 08 | 2 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 08 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 08. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 09    Communications Test

Format:        <SI>09[Data]<SO>[LRC] (request frame)

<SI>09[Data]<SO>[LRC] (response frame)

Message length:  Fixed.

Usage:           Performs a two-step communication loopback test.

Implementation:  The host must echo the first response payload. The device then returns status 0 for success.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 09 | 2 | Message ID |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 09 | 2 | Message ID |
| <SUB> | 1 | Subfield delimiter. |
| PROCESSING | 10 | Processing echo text, followed by final status in the second exchange. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 09 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 09. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 11    Device Connection Test

Format:        <SI>11[Data]<SO>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Connection probe. A valid frame is acknowledged without a final EOT.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 11 | 2 | Message ID |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 11 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message 12    Select Prompt Language

Format:        <SI>12[Data]<SO>[LRC] (request frame)

<SI>12[Data]<SO>[LRC] (response frame)

Message length:  Fixed.

Usage:           Compatibility prompt-language command.

Implementation:  The Android application language follows Android locale settings.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 12 | 2 | Message ID |
| [Language index] | 1 | Language table index requested by host. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 12 | 2 | Message ID |
| [Status] | 1 | '0' accepted, '1' rejected. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 12 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 12. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 13    Set Communication Speed

Format:        <SI>13[Data]<SO>[LRC] (request frame)

<SI>13[Data]<SO>[LRC] (response frame)

Message length:  Fixed.

Usage:           Validates a communication speed request.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 13 | 2 | Message ID |
| [Baud code] | 1 | Baud-rate selector 1 through 8. |
| [Mode] | 0 or 1 | Optional communication mode selector. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 13 | 2 | Message ID |
| [Status] | 1 | '0' accepted, '1' rejected. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 13 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 13. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 14    Acknowledge Without Final End

Format:        <SI>14[Data]<SO>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Compatibility acknowledgement command.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 14 | 2 | Message ID |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 14 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message 17    Request Random Number

Format:        <SI>17[Data]<SO>[LRC] (request frame)

<SI>17[Data]<SO>[LRC] (response frame)

Message length:  Fixed.

Usage:           Returns an 8-byte random challenge.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 17 | 2 | Message ID |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 17 | 2 | Message ID |
| [Random] | 16 | Eight random bytes encoded as hexadecimal. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 17 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 17. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 19    Query Firmware Version

Format:        <SI>19[Data]<SO>[LRC] (request frame)

<SI>19[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns component firmware or application version.

Implementation:  The version string is formatted for compatibility and includes the NEXGO model when available.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 19 | 2 | Message ID |
| [Component] | 1 | Firmware component selector. |
| [Option] | 0 or 1 | Optional component option. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 19 | 2 | Message ID |
| . | 1 | Version prefix separator. |
| [Version] | Variable | Application, SDK, or device firmware version string. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 19 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 19. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 1C    Query Hardware Capability

Format:        <SI>1C[Data]<SO>[LRC] (request frame)

<SI>1C[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns supported hardware and application capability values.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 1C | 2 | Message ID |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 1C | 2 | Message ID |
| <FS> | 1 | Field separator before each capability item. |
| [Capability] | Variable | Capability string. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 1C request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 1C. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 1F    Query Prompt Table

Format:        <SI>1F[Data]<SO>[LRC] (request frame)

<SI>1F[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Reports usable prompt tables.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 1F | 2 | Message ID |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 1F | 2 | Message ID |
| [Language list] | Variable | Prompt-language identifiers separated by <FS>. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 1F request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 1F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 1M    Setup Keypad Beeper

Format:        <SI>1M[Data]<SO>[LRC] (request frame)

<SI>1M[Data]<SO>[LRC] (response frame)

Message length:  Fixed.

Usage:           Controls keypad beep preference.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 1M | 2 | Message ID |
| [Option] | 1 | '0' disable, '1' enable. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 1M | 2 | Message ID |
| [Status] | 1 | Echoed option on success. '0' on invalid request. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 1M request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 1M. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message 1P    Control Beeper

Format:        <SI>1P[Data]<SO>[LRC] (request frame)

<SI>1P[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Plays one or more beeps.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 1P | 2 | Message ID |
| <SUB> | 1 | Subfield delimiter. |
| [Count] | 1 | Number of beeps, 1 through 9. |
| <FS> | 1 | Field separator. |
| [Duration] | 1 to 3 | Duration in 10 ms units. |
| [Interval] | 1 to 3 | Interval in 10 ms units. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| 1P | 2 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 1P request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 1P. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message M03    Load Permanent Unit Serial Number

Format:        <SI>M03[Data]<SO>[LRC] (request frame)

<SI>M03[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores a reinstall-persistent private serial number.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| M03 | 3 | Message ID |
| [Serial] | 11 | Uppercase alphanumeric or dash serial value. |
| [Slot] | 0 or 1 | Optional slot 0 through 7. Default 0. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| M03 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' bad serial, '2' already set, '3' storage or slot error. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message M03 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message M03. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

Message M04    Query Permanent Unit Serial Number

Format:        <SI>M04[Data]<SO>[LRC] (request frame)

<SI>M04[Data]<SO>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns the permanent unit serial number.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| M04 | 3 | Message ID |
| [Slot] | 0 or 1 | Optional slot. Default 0. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <SI> | 1 | <0F> |
| M04 | 3 | Message ID |
| [Serial] | 0 or 11 | Stored permanent serial, empty if none was loaded. |
| <SO> | 1 | <0E> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message M04 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message M04. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |
|  | <- | Send final <EOT> after host <ACK>. |

# Section 3    Security and PIN Messages

Message 20    Load Secret Master Key

Format:        <STX>20[Data]<ETX>[LRC] (request frame)

<STX>20[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Loads the secret master key used by secret PIN session slots.

Implementation:  The key is injected into internal secure slots 20 through 29. Session-key loading waits until this master key is ready.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 20 | 2 | Message ID |
| [Option] | 1 | '0' or '1'. |
| [Key value] | 16 or 32 | Secret master key in hexadecimal. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 20 | 2 | Message ID |
| 20 | 2 | Message ID echoed with original payload on success. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 20 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 20. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 21    Load Secret Session Key

Format:        <STX>21[Data]<ETX>[LRC] (request frame)

<STX>21[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Loads a secret MK/SK session key for PIN entry.

Implementation:  Key id 0 through 9 maps internally to secure slots 20 through 29.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 21 | 2 | Message ID |
| [Key ID] | 1 | Secret session key id 0 through 9. |
| [Key value] | 16 or 32 | Encrypted PIN session key in hexadecimal. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 21 | 2 | Message ID |
| 21 | 2 | Message ID echoed with original payload on success. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 21 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 21. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 22    PIN Entry Request Using Secret MK/SK

Format:        <STX>22[Data]<ETX>[LRC] (request frame)

<STX>71[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts secure online PIN entry using the secret key family.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 22 | 2 | Message ID |
| [Account] | 8 to 19 | PAN/account digits used for PIN block formatting. |
| <FS> | 1 | Field separator. |
| [Key/control] | Variable | Secret key id followed by optional PIN control and prompt data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [Status/PIN data] | Variable | Message 71 payload containing PIN block or error code. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 22 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 71. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 23    PIN Entry Request With External Prompt

Format:        <STX>23[Data]<ETX>[LRC] (request frame)

<STX>71[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts secure online PIN entry using the secret key family.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 23 | 2 | Message ID |
| [Account] | 8 to 19 | PAN/account digits used for PIN block formatting. |
| <FS> | 1 | Field separator. |
| [Key/control] | Variable | Secret key id followed by optional PIN control and prompt data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [Status/PIN data] | Variable | Message 71 payload containing PIN block or error code. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 23 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 71. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 24    PIN Entry Request With Custom Prompt

Format:        <STX>24[Data]<ETX>[LRC] (request frame)

<STX>71[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts secure online PIN entry using the secret key family.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 24 | 2 | Message ID |
| [Account] | 8 to 19 | PAN/account digits used for PIN block formatting. |
| <FS> | 1 | Field separator. |
| [Key/control] | Variable | Secret key id followed by optional PIN control and prompt data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [Status/PIN data] | Variable | Message 71 payload containing PIN block or error code. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 24 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 71. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 60    DUKPT PIN Entry Request

Format:        <STX>60[Data]<ETX>[LRC] (request frame)

<STX>71[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts online PIN entry using DUKPT.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 60 | 2 | Message ID |
| [Account] | 8 to 19 | PAN/account digits. |
| <FS> | 1 | Field separator. |
| [Session key/control] | Variable | Session key, PIN length control, optional prompts, and optional timeout. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [PIN response] | Variable | Message 71 payload. Includes status, PIN block, KSN/key id, or error. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 60 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 71. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 70    PIN Entry Request

Format:        <STX>70[Data]<ETX>[LRC] (request frame)

<STX>71[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts DUKPT or MK/SK PIN entry. A leading dot selects MK/SK.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 70 | 2 | Message ID |
| [Account] | 8 to 19 | PAN/account digits. |
| <FS> | 1 | Field separator. |
| [Session key/control] | Variable | Session key, PIN length control, optional prompts, and optional timeout. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [PIN response] | Variable | Message 71 payload. Includes status, PIN block, KSN/key id, or error. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 70 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 71. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 7G    PIN Change Request

Format:        <STX>7G[Data]<ETX>[LRC] (request frame)

<STX>71[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts MK/SK PIN change style entry.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 7G | 2 | Message ID |
| [Account] | 8 to 19 | PAN/account digits. |
| <FS> | 1 | Field separator. |
| [Session key/control] | Variable | Session key, PIN length control, optional prompts, and optional timeout. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [PIN response] | Variable | Message 71 payload. Includes status, PIN block, KSN/key id, or error. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 7G request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 71. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z60    External Prompt PIN Entry

Format:        <STX>Z60[Data]<ETX>[LRC] (request frame)

<STX>71[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts PIN entry using an external prompt.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z60 | 3 | Message ID |
| [Account] | 8 to 19 | PAN/account digits. |
| <FS> | 1 | Field separator. |
| [Session key/control] | Variable | Session key, PIN length control, optional prompts, and optional timeout. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [PIN response] | Variable | Message 71 payload. Includes status, PIN block, KSN/key id, or error. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z60 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 71. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z62    Custom Prompt PIN Entry

Format:        <STX>Z62[Data]<ETX>[LRC] (request frame)

<STX>71[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts PIN entry with custom prompt lines and PIN limits.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z62 | 3 | Message ID |
| [Account] | 8 to 19 | PAN/account digits. |
| <FS> | 1 | Field separator. |
| [Session key/control] | Variable | Session key, PIN length control, optional prompts, and optional timeout. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [PIN response] | Variable | Message 71 payload. Includes status, PIN block, KSN/key id, or error. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z62 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 71. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 62    Amount Authorization

Format:        <STX>62[Data]<ETX>[LRC] (request frame)

<STX>63[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Compatibility command that returns message 63.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 62 | 2 | Message ID |
| [Data] | Variable | Compatibility payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 63 | 2 | Message ID |
| [Status] | 1 | '0' accepted. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 62 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 63. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 63    Response Message Compatibility Acknowledgement

Format:        <STX>63[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Accepted as a no-op when a host sends a response-id command.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 63 | 2 | Message ID |
| [Payload] | Variable | Ignored when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 63 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message 71    Response Message Compatibility Acknowledgement

Format:        <STX>71[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Accepted as a no-op when a host sends a response-id command.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 71 | 2 | Message ID |
| [Payload] | Variable | Ignored when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 71 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message 91    Response Message Compatibility Acknowledgement

Format:        <STX>91[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Accepted as a no-op when a host sends a response-id command.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 91 | 2 | Message ID |
| [Payload] | Variable | Ignored when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 91 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message 99    Response Message Compatibility Acknowledgement

Format:        <STX>99[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Accepted as a no-op when a host sends a response-id command.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 99 | 2 | Message ID |
| [Payload] | Variable | Ignored when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 99 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message Z65    Response Message Compatibility Acknowledgement

Format:        <STX>Z65[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Accepted as a no-op when a host sends a response-id command.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z65 | 3 | Message ID |
| [Payload] | Variable | Ignored when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z65 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message Z67    Response Message Compatibility Acknowledgement

Format:        <STX>Z67[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Accepted as a no-op when a host sends a response-id command.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z67 | 3 | Message ID |
| [Payload] | Variable | Ignored when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z67 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message 7A    Set DUKPT KSN Output Format

Format:        <STX>7A[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Controls DUKPT KSN formatting.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 7A | 2 | Message ID |
| [Format] | 1 | KSN output format selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 7A request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message 90    Load DUKPT Initial Key Set 0

Format:        <STX>90[Data]<ETX>[LRC] (request frame)

<STX>91[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Injects the first DUKPT key set.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 90 | 2 | Message ID |
| [IPEK] | 32 | DUKPT initial key. |
| [KSN] | 20 | Initial key serial number. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 91 | 2 | Message ID |
| [Status] | 1 | '0' success. '1' followed by reason on failure. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 90 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 91. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 94    Load DUKPT Initial Key Set 1

Format:        <STX>94[Data]<ETX>[LRC] (request frame)

<STX>91[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Injects the second DUKPT key set.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 94 | 2 | Message ID |
| [IPEK] | 32 | DUKPT initial key. |
| [KSN] | 20 | Initial key serial number. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 91 | 2 | Message ID |
| [Status] | 1 | '0' success. '1' followed by reason on failure. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 94 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 91. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 96    Select DUKPT Key Set

Format:        <STX>96[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Selects active DUKPT key set.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 96 | 2 | Message ID |
| [Key set] | 1 | '0' or '1'. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 96 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message 98    Query DUKPT Key Set

Format:        <STX>98[Data]<ETX>[LRC] (request frame)

<STX>99[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Queries DUKPT key state.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 98 | 2 | Message ID |
| [Key set] | 1 | '0' or '1'. |
| [Info flag] | 0 or 1 | Optional '1' requests extra key information. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 99 | 2 | Message ID |
| [Status] | 1 | Key state. |
| [Info] | Variable | Optional algorithm/type/KCV style fields separated by <FS>. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 98 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 99. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z64    Query Master Key KCV

Format:        <STX>Z64[Data]<ETX>[LRC] (request frame)

<STX>Z65[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Calculates the master key check value.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z64 | 3 | Message ID |
| [Key ID] | 1 | Master key id. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z65 | 3 | Message ID |
| [Key ID] | 1 | Echoed key id. |
| [KCV] | 6 | Three-byte key check value, or '?' when unavailable. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z64 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z65. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z66    Generate MAC

Format:        <STX>Z66[Data]<ETX>[LRC] (request frame)

<STX>Z67[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Generates a MAC using the active MAC-capable key.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z66 | 3 | Message ID |
| [MAC data] | Variable | Data to MAC according to active key metadata. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z67 | 3 | Message ID |
| [Status] | 1 | '0' success, '2' failure. |
| [MAC] | 16 | MAC value when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z66 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z67. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

# Section 4    Magnetic Stripe and Multi-Interface Messages

Message Q1    Arm Magnetic Stripe Reader

Format:        <STX>Q1[Data]<ETX>[LRC] (request frame)

<STX>81[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Displays the swipe prompt and waits for a swipe. Cached multi-interface swipe data is returned immediately when available.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q1 | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 81 | 2 | Message ID |
| [Track 1] | Variable | Track 1 data when present. |
| <FS> | 1 | Field separator. |
| [Track 2] | Variable | Track 2 data when present. |
| [Track 3] | Variable | Track 3 data when present. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q1 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 81. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Q8    Read Contactless Track Data

Format:        <STX>Q8[Data]<ETX>[LRC] (request frame)

<STX>83[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts RF card read and returns track-equivalent data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q8 | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 83 | 2 | Message ID |
| [Track 1] | Variable | Track 1 data when present. |
| <FS> | 1 | Field separator. |
| [Track 2] | Variable | Track 2 data when present. |
| [Track 3] | Variable | Track 3 data when present. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q8 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 83. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Q9    Read Swipe or Contactless Track Data

Format:        <STX>Q9[Data]<ETX>[LRC] (request frame)

<STX>81/83[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Arms both swipe and RF track paths. The first successful reader wins.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q9 | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 81/83 | 5 | Message ID |
| [Track 1] | Variable | Track 1 data when present. |
| <FS> | 1 | Field separator. |
| [Track 2] | Variable | Track 2 data when present. |
| [Track 3] | Variable | Track 3 data when present. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q9 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 81/83. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message QF    Arm Magnetic Stripe Reader With Image Prompt

Format:        <STX>QF[Data]<ETX>[LRC] (request frame)

<STX>81[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Compatibility image-prompt variant. Android prompt animation is used.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QF | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 81 | 2 | Message ID |
| [Track 1] | Variable | Track 1 data when present. |
| <FS> | 1 | Field separator. |
| [Track 2] | Variable | Track 2 data when present. |
| [Track 3] | Variable | Track 3 data when present. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QF request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 81. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message QG    Read Contactless Track Data With Image Prompt

Format:        <STX>QG[Data]<ETX>[LRC] (request frame)

<STX>83[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Compatibility image-prompt variant. Android prompt animation is used.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QG | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 83 | 2 | Message ID |
| [Track 1] | Variable | Track 1 data when present. |
| <FS> | 1 | Field separator. |
| [Track 2] | Variable | Track 2 data when present. |
| [Track 3] | Variable | Track 3 data when present. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QG request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 83. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message QH    Read Swipe or Contactless Track Data With Image Prompt

Format:        <STX>QH[Data]<ETX>[LRC] (request frame)

<STX>81/83[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Compatibility dual-reader image-prompt variant.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QH | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 81/83 | 5 | Message ID |
| [Track 1] | Variable | Track 1 data when present. |
| <FS> | 1 | Field separator. |
| [Track 2] | Variable | Track 2 data when present. |
| [Track 3] | Variable | Track 3 data when present. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QH request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message 81/83. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Q2    Transaction Complete

Format:        <STX>Q2[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Completes a magnetic stripe/card-read session and returns to idle.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q2 | 2 | Message ID |
| [Option] | Variable | Command-specific option. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q2 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message Q3    Ignore Card Swipe

Format:        <STX>Q3[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Stops active swipe handling and ignores the current swipe.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q3 | 2 | Message ID |
| [Option] | Variable | Command-specific option. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q3 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message Q4    Set Swipe Track Mode

Format:        <STX>Q4[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Sets swipe track mode: 0 track 2 only, 1 disabled, 2 all tracks.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q4 | 2 | Message ID |
| [Option] | Variable | Command-specific option. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q4 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message Q5    Set Swipe Retry Count

Format:        <STX>Q5[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Sets retry count 0 through 9.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q5 | 2 | Message ID |
| [Option] | Variable | Command-specific option. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q5 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message QA    Set Contactless Track Mode

Format:        <STX>QA[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Sets contactless track mode: 0 track 2 only, 1 disabled, 2 all tracks.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QA | 2 | Message ID |
| [Option] | Variable | Command-specific option. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QA request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message Q6    Set Swipe Output Format

Format:        <STX>Q6[Data]<ETX>[LRC] (request frame)

<STX>Q6[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Sets magnetic stripe output format.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q6 | 2 | Message ID |
| [Format] | 1 | '0' clear text, '2' clear text with sentinels. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q6 | 2 | Message ID |
| Q6 | 2 | Request frame echoed on success. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q6 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Q6. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Q7    Query Swipe Output Format

Format:        <STX>Q7[Data]<ETX>[LRC] (request frame)

<STX>Q7[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Returns magnetic stripe output format.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q7 | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Q7 | 2 | Message ID |
| [Format] | 1 | Current output format. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Q7 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Q7. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message QB    Set Swipe Auto Arm

Format:        <STX>QB[Data]<ETX>[LRC] (request frame)

<STX>QB[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Enables or disables automatic swipe arming.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QB | 2 | Message ID |
| [Option] | 1 | '0' disable, '1' enable. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QB | 2 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QB request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message QB. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message QC    Set Contactless Auto Arm

Format:        <STX>QC[Data]<ETX>[LRC] (request frame)

<STX>QC[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Enables or disables contactless auto-arm compatibility flag.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QC | 2 | Message ID |
| [Option] | 1 | '0' disable, '1' enable. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QC | 2 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QC request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message QC. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message QD    Set Contactless Output Format

Format:        <STX>QD[Data]<ETX>[LRC] (request frame)

<STX>QD[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Sets contactless track-equivalent output format.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QD | 2 | Message ID |
| [Format] | 1 | '0' clear text, '2' clear text with sentinels. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QD | 2 | Message ID |
| QD | 2 | Request frame echoed on success. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QD request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message QD. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message QI    Query Contactless Tags

Format:        <STX>QI[Data]<ETX>[LRC] (request frame)

<STX>QJ[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns requested contactless transaction data objects.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QI | 2 | Message ID |
| [DOL] | Variable | Tag list or data object list. Optional ready-control byte may precede it. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QJ | 2 | Message ID |
| [Status/control] | Variable | 'F' for no data or '11' plus TLV data. |
| <FS> | 1 | Field separator before TLV data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QI request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message QJ. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message QK    Multi-Interface Card Detection

Format:        <STX>QK[Data]<ETX>[LRC] (request frame)

<STX>QK[Data]<ETX>[LRC] (response frame)

Message length:  Fixed.

Usage:           Prompts for swipe, insert, or tap and returns the detected interface.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QK | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| QK | 2 | Message ID |
| [Result] | 1 | '1' swipe, '2' chip, '3' contactless, '0' error/cancel. |
| [Error] | 0 or 1 | Error code when result is 0. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message QK request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message QK. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 81    Magnetic Stripe Data

Format:        <STX>81[Data]<ETX>[LRC] (request frame)

<STX>81[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Device-to-host track-data response.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 81 | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 81 | 2 | Message ID |
| [Track data] | Variable | Track 1, track 2, and track 3 separated by <FS>. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 81 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 81. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 83    Contactless Track Data

Format:        <STX>83[Data]<ETX>[LRC] (request frame)

<STX>83[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Device-to-host RF track-equivalent response.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 83 | 2 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 83 | 2 | Message ID |
| [Track data] | Variable | Track-equivalent data separated by <FS>. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 83 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 83. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

# Section 5    CPU Smart Card Messages

Message I00    Query Primary Smart Card Presence

Format:        <STX>I00[Data]<ETX>[LRC] (request frame)

<STX>I00[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Checks ICC card presence.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I00 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I00 | 3 | Message ID |
| [Status] | 1 | 'F' present, '0' absent. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I00 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I00. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I01    Primary Smart Card Cold Reset

Format:        <STX>I01[Data]<ETX>[LRC] (request frame)

<STX>I02[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Powers on ICC card and returns ATR.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I01 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I02 | 3 | Message ID |
| [ATR] | Variable | ATR returned in message I02. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I01 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I02. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I04    Primary Smart Card Deactivate

Format:        <STX>I04[Data]<ETX>[LRC] (request frame)

<STX>I04[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Powers off ICC card.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I04 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I04 | 3 | Message ID |
| [Status] | 0 | No data on success. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I04 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I04. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I06    Primary Smart Card APDU Exchange

Format:        <STX>I06[Data]<ETX>[LRC] (request frame)

<STX>I07[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Exchanges an APDU with the powered ICC card.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I06 | 3 | Message ID |
| [APDU] | 8 to 524 | APDU command in hexadecimal. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I07 | 3 | Message ID |
| [APDU response] | Variable | Response APDU returned in message I07. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I06 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I07. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I11    SAM Cold Reset

Format:        <STX>I11[Data]<ETX>[LRC] (request frame)

<STX>I12[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Powers on selected SAM and returns ATR.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I11 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I12 | 3 | Message ID |
| [ATR] | Variable | ATR returned in message I12. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I11 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I12. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I14    SAM Deactivate

Format:        <STX>I14[Data]<ETX>[LRC] (request frame)

<STX>I14[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Powers off selected SAM.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I14 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I14 | 3 | Message ID |
| [Status] | 0 | No data on success. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I14 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I14. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I15    Select SAM Interface

Format:        <STX>I15[Data]<ETX>[LRC] (request frame)

<STX>I15[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Selects active SAM slot.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I15 | 3 | Message ID |
| [Interface] | 1 | '1' PSAM1, '2' PSAM2. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I15 | 3 | Message ID |
| [Interface] | 1 | Selected interface echoed. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I15 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I15. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I16    SAM APDU Exchange

Format:        <STX>I16[Data]<ETX>[LRC] (request frame)

<STX>I17[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Exchanges an APDU with the powered SAM.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I16 | 3 | Message ID |
| [APDU] | 8 to 524 | APDU command in hexadecimal. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I17 | 3 | Message ID |
| [APDU response] | Variable | Response APDU returned in message I17. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I16 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I17. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I02    CPU Card Compatibility or Response Identifier

Format:        <STX>I02[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I02 | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I02 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I03    CPU Card Compatibility or Response Identifier

Format:        <STX>I03[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I03 | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I03 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I05    CPU Card Compatibility or Response Identifier

Format:        <STX>I05[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I05 | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I05 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I07    CPU Card Compatibility or Response Identifier

Format:        <STX>I07[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I07 | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I07 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I08    CPU Card Compatibility or Response Identifier

Format:        <STX>I08[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I08 | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I08 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I09    CPU Card Compatibility or Response Identifier

Format:        <STX>I09[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I09 | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I09 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I0A    CPU Card Compatibility or Response Identifier

Format:        <STX>I0A[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0A | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I0A request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I0B    CPU Card Compatibility or Response Identifier

Format:        <STX>I0B[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0B | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I0B request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I0C    CPU Card Compatibility or Response Identifier

Format:        <STX>I0C[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0C | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I0C request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I0D    CPU Card Compatibility or Response Identifier

Format:        <STX>I0D[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0D | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I0D request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I0E    CPU Card Compatibility or Response Identifier

Format:        <STX>I0E[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0E | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I0E request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I0F    CPU Card Compatibility or Response Identifier

Format:        <STX>I0F[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I0F request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I12    CPU Card Compatibility or Response Identifier

Format:        <STX>I12[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I12 | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I12 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message I17    CPU Card Compatibility or Response Identifier

Format:        <STX>I17[Data]<ETX>[LRC] (request frame)

<STX>I0F[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reserved, unsupported, or device response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I17 | 3 | Message ID |
| [Payload] | Variable | Ignored or unsupported when received from host. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| I0F | 3 | Message ID |
| [Error] | 2 | I0F error code for unsupported host request. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message I17 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message I0F. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

# Section 6    RF Memory Card Messages

Message P01    Enable or Disable RF Memory Card Reader

Format:        <STX>P01[Data]<ETX>[LRC] (request frame)

<STX>P01[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Opens or closes the RF reader.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P01 | 3 | Message ID |
| [Data] | Variable | [0/1] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P01 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P01 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P01. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P02    Query RF Card Presence

Format:        <STX>P02[Data]<ETX>[LRC] (request frame)

<STX>P02[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns ATQA when a Type A card is present.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P02 | 3 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P02 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P02 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P02. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P03    RF Anticollision

Format:        <STX>P03[Data]<ETX>[LRC] (request frame)

<STX>P03[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns UID.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P03 | 3 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P03 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P03 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P03. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P04    RF Select

Format:        <STX>P04[Data]<ETX>[LRC] (request frame)

<STX>P04[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns SAK.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P04 | 3 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P04 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P04 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P04. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P05    RF Activate

Format:        <STX>P05[Data]<ETX>[LRC] (request frame)

<STX>P05[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns ATQA, SAK, and UID.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P05 | 3 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P05 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P05 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P05. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P06    RF Halt

Format:        <STX>P06[Data]<ETX>[LRC] (request frame)

<STX>P06[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Halts or deselects the current card.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P06 | 3 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P06 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P06 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P06. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P07    Authenticate Classic Sector

Format:        <STX>P07[Data]<ETX>[LRC] (request frame)

<STX>P07[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Authenticates a Classic sector using direct or stored key.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P07 | 3 | Message ID |
| [Data] | Variable | [Sector][Key data] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P07 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P07 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P07. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P08    Read Ultralight Page

Format:        <STX>P08[Data]<ETX>[LRC] (request frame)

<STX>P08[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reads a 4-byte Ultralight page.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P08 | 3 | Message ID |
| [Data] | Variable | [Page] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P08 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P08 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P08. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P09    Write Ultralight Page

Format:        <STX>P09[Data]<ETX>[LRC] (request frame)

<STX>P09[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Writes a 4-byte Ultralight page.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P09 | 3 | Message ID |
| [Data] | Variable | [Page][Data] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P09 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P09 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P09. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P10    Read Block

Format:        <STX>P10[Data]<ETX>[LRC] (request frame)

<STX>P10[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reads a block or page.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P10 | 3 | Message ID |
| [Data] | Variable | [Block] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P10 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P10 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P10. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P11    Write Block

Format:        <STX>P11[Data]<ETX>[LRC] (request frame)

<STX>P11[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Writes a block or page.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P11 | 3 | Message ID |
| [Data] | Variable | [Block][Data] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P11 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P11 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P11. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P12    Read Sector

Format:        <STX>P12[Data]<ETX>[LRC] (request frame)

<STX>P12[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Reads all blocks in a sector.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P12 | 3 | Message ID |
| [Data] | Variable | [Sector] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P12 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P12 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P12. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P13    Write Sector

Format:        <STX>P13[Data]<ETX>[LRC] (request frame)

<STX>P13[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Writes sequential blocks in a sector.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P13 | 3 | Message ID |
| [Data] | Variable | [Sector][Data] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P13 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P13 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P13. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P14    Value Operation

Format:        <STX>P14[Data]<ETX>[LRC] (request frame)

<STX>P14[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Performs value write, increment, decrement, or backup operation.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P14 | 3 | Message ID |
| [Data] | Variable | [Block][Mode][Value/Target] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P14 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P14 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P14. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P15    Load RF Key

Format:        <STX>P15[Data]<ETX>[LRC] (request frame)

<STX>P15[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores key A and key B for later authentication.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P15 | 3 | Message ID |
| [Data] | Variable | [KeyNo][KeyA][KeyB] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P15 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P15 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P15. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P16    Identify RF Card Type

Format:        <STX>P16[Data]<ETX>[LRC] (request frame)

<STX>P16[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns card type code.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P16 | 3 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P16 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P16 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P16. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P17    Activate DESFire

Format:        <STX>P17[Data]<ETX>[LRC] (request frame)

<STX>P17[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Activates DESFire and returns ATS or UID.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P17 | 3 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P17 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P17 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P17. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P18    Deselect DESFire

Format:        <STX>P18[Data]<ETX>[LRC] (request frame)

<STX>P18[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Deselects DESFire and clears RF state.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P18 | 3 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P18 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P18 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P18. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P19    RF APDU Exchange

Format:        <STX>P19[Data]<ETX>[LRC] (request frame)

<STX>P19[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Exchanges APDU or raw command depending on detected RF type.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P19 | 3 | Message ID |
| [Data] | Variable | [APDU] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P19 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P19 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P19. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message P20    RF Raw Block Exchange

Format:        <STX>P20[Data]<ETX>[LRC] (request frame)

<STX>P20[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Performs raw RF command exchange.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P20 | 3 | Message ID |
| [Data] | Variable | [CRC][Wait][Data] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| P20 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Data] | Variable | Command-specific response data when successful. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message P20 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message P20. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

# Section 7    Display, Prompt, Data Entry, and Image Messages

Message B1    Set Display Font Size

Format:        <STX>B1[Data]<ETX>[LRC] (request frame)

<STX>B2[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Sets display font size compatibility option.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| B1 | 2 | Message ID |
| [Data] | Variable | [Option] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| B2 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message B1 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message B2. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message B3    Set Display Font Color

Format:        <STX>B3[Data]<ETX>[LRC] (request frame)

<STX>B4[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Sets display font color compatibility option.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| B3 | 2 | Message ID |
| [Data] | Variable | [Foreground][Background] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| B4 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message B3 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message B4. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message BB    Set Screen Saver

Format:        <STX>BB[Data]<ETX>[LRC] (request frame)

<STX>BC[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Sets screen saver compatibility option.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| BB | 2 | Message ID |
| [Data] | Variable | [WaitTime][Type] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| BC | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message BB request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message BC. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message BD    Enable or Disable Screen Saver

Format:        <STX>BD[Data]<ETX>[LRC] (request frame)

<STX>BE[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Enables or disables screen saver compatibility option.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| BD | 2 | Message ID |
| [Data] | Variable | [0/1] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| BE | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message BD request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message BE. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J0    Initialize Image Table

Format:        <STX>J0[Data]<ETX>[LRC] (request frame)

<STX>J0[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Initializes local image table.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J0 | 2 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J0 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J0 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J0. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J1    Query Image Table

Format:        <STX>J1[Data]<ETX>[LRC] (request frame)

<STX>J1[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns image table entries.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J1 | 2 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J1 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J1 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J1. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J2    Select Images

Format:        <STX>J2[Data]<ETX>[LRC] (request frame)

<STX>J2[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Selects stored image names.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J2 | 2 | Message ID |
| [Data] | Variable | [Control][Names] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J2 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J2 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J2. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J3    Delete Images

Format:        <STX>J3[Data]<ETX>[LRC] (request frame)

<STX>J3[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Deletes stored images.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J3 | 2 | Message ID |
| [Data] | Variable | [Names] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J3 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J3 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J3. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J4    Download Image Packet

Format:        <STX>J4[Data]<ETX>[LRC] (request frame)

<STX>J4[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores image packet data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J4 | 2 | Message ID |
| [Data] | Variable | [Packet] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J4 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J4 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J4. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J5    Upload Image Packet

Format:        <STX>J5[Data]<ETX>[LRC] (request frame)

<STX>J5[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns stored image packet data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J5 | 2 | Message ID |
| [Data] | Variable | [Control][Name] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J5 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J5 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J5. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J7    Set Idle Image

Format:        <STX>J7[Data]<ETX>[LRC] (request frame)

<STX>J7[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Accepted for compatibility.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J7 | 2 | Message ID |
| [Data] | Variable | [Name] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J7 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J7 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J7. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J8    Enable Idle Image

Format:        <STX>J8[Data]<ETX>[LRC] (request frame)

<STX>J8[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Accepted for compatibility.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J8 | 2 | Message ID |
| [Data] | Variable | [Option] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J8 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J8 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J8. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message J9    Show Image

Format:        <STX>J9[Data]<ETX>[LRC] (request frame)

<STX>J9[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Shows stored image if available.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J9 | 2 | Message ID |
| [Data] | Variable | [Name] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J9 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J9 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message J9. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message JA    Download Boot Logo

Format:        <STX>JA[Data]<ETX>[LRC] (request frame)

<STX>JA[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores boot logo packet data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| JA | 2 | Message ID |
| [Data] | Variable | [Packet] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| JA | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status or data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message JA request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message JA. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message BF    Preview Screen Saver

Format:        <STX>BF[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Compatibility no-op.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| BF | 2 | Message ID |
| [Data] | Variable | Command-specific payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message BF request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message J6    Play Selected Images

Format:        <STX>J6[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Variable.

Usage:           Displays selected image sequence if available.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| J6 | 2 | Message ID |
| [Data] | Variable | Command-specific payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message J6 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message Z0    Move Display Cursor

Format:        <STX>Z0[Data]<ETX>[LRC] (request frame)

<STX>Z0[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Compatibility cursor command.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z0 | 2 | Message ID |
| [Data] | Variable | [Line] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z0 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z0 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z0. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z1    Reset Display and Transaction State

Format:        <STX>Z1[Data]<ETX>[LRC] (request frame)

<STX>Z1[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Clears prompts, transaction amount display, cached card data, active reads, and active PIN entry.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z1 | 2 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z1 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z1 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z1. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message 72    Cancel Transaction

Format:        <STX>72[Data]<ETX>[LRC] (request frame)

<STX>72[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Cancels active card or PIN operation and returns to idle.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 72 | 2 | Message ID |
| [Data] | Variable | No payload. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| 72 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message 72 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message 72. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z2    Display Prompt

Format:        <STX>Z2[Data]<ETX>[LRC] (request frame)

<STX>Z2[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Displays one fixed or custom prompt.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z2 | 2 | Message ID |
| [Data] | Variable | [Prompt] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z2 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z2 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z2. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z3    Display Multi-Line Prompt

Format:        <STX>Z3[Data]<ETX>[LRC] (request frame)

<STX>Z3[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Displays fixed or custom prompt lines.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z3 | 2 | Message ID |
| [Data] | Variable | [Prompts] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z3 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z3 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z3. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z7    Cancel Message Display Option

Format:        <STX>Z7[Data]<ETX>[LRC] (request frame)

<STX>Z7[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Controls whether cancel message is shown.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z7 | 2 | Message ID |
| [Data] | Variable | [0/1] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z7 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z7 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z7. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z8    Set Idle Prompt

Format:        <STX>Z8[Data]<ETX>[LRC] (request frame)

<STX>Z8[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Sets the idle prompt text.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z8 | 2 | Message ID |
| [Data] | Variable | [Prompt] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z8 | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z8 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z8. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message ZA    Set Transaction Display Context

Format:        <STX>ZA[Data]<ETX>[LRC] (request frame)

<STX>ZA[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores transaction amount, currency, and transaction type for the next card-read flow.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| ZA | 2 | Message ID |
| [Data] | Variable | [Type]<FS>[Amount]<FS>[Currency]<FS>[Symbol] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| ZA | 2 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message ZA request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message ZA. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z42    Read Key Code

Format:        <STX>Z42[Data]<ETX>[LRC] (request frame)

<STX>Z43[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts keypad single-key read after a prompt.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z42 | 3 | Message ID |
| [Data] | Variable | [Timeout] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z43 | 3 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z42 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message Z43. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z50    String Entry

Format:        <STX>Z50[Data]<ETX>[LRC] (request frame)

<STX>Z51[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts keypad string entry after a prompt.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z50 | 3 | Message ID |
| [Data] | Variable | [Entry control] |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z51 | 3 | Message ID |
| [Status/Data] | Variable | Command-specific status, key code, text, or no frame depending on command. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z50 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message Z51. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z43    Data Entry Response Identifier

Format:        <STX>Z43[Data]<ETX>[LRC] (request frame)

<STX>Z43[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Device sends this after data entry completes.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z43 | 3 | Message ID |
| [Payload] | Variable | Device-to-host response identifier. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z43 | 3 | Message ID |
| [Payload] | Variable | Key code or entered text. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z43 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z43. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message Z51    Data Entry Response Identifier

Format:        <STX>Z51[Data]<ETX>[LRC] (request frame)

<STX>Z51[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Device sends this after data entry completes.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z51 | 3 | Message ID |
| [Payload] | Variable | Device-to-host response identifier. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| Z51 | 3 | Message ID |
| [Payload] | Variable | Key code or entered text. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message Z51 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message Z51. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

# Section 8    EMV Contact and Contactless Messages

Message T01    Load Contact Terminal Configuration

Format:        <STX>T01[Data]<ETX>[LRC] (request frame)

<STX>T02[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores terminal configuration data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T01 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T02 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T01 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T02. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T03    Load Contact CAPK

Format:        <STX>T03[Data]<ETX>[LRC] (request frame)

<STX>T04[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores certification authority public key data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T03 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T04 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T03 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T04. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T05    Load Contact Application Configuration

Format:        <STX>T05[Data]<ETX>[LRC] (request frame)

<STX>T06[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores contact application configuration.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T05 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T06 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T05 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T06. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T07    Load EMV Data Format Table

Format:        <STX>T07[Data]<ETX>[LRC] (request frame)

<STX>T08[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores tag format definitions for private and standard tags.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T07 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T08 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T07 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T08. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T09    Query Contact Configuration IDs

Format:        <STX>T09[Data]<ETX>[LRC] (request frame)

<STX>T0A[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns stored contact configuration identifiers.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T09 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T0A | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T09 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T0A. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T0B    Delete Contact Configuration

Format:        <STX>T0B[Data]<ETX>[LRC] (request frame)

<STX>T0C[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Deletes stored contact configuration.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T0B | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T0C | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T0B request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T0C. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T51    Load Contactless Terminal Configuration

Format:        <STX>T51[Data]<ETX>[LRC] (request frame)

<STX>T52[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores contactless terminal configuration.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T51 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T52 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T51 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T52. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T53    Load Contactless CAPK

Format:        <STX>T53[Data]<ETX>[LRC] (request frame)

<STX>T54[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores contactless CAPK only when not already present.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T53 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T54 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T53 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T54. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T55    Load Contactless Application Configuration

Format:        <STX>T55[Data]<ETX>[LRC] (request frame)

<STX>T56[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores contactless application configuration.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T55 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T56 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T55 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T56. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T59    Query Contactless Configuration IDs

Format:        <STX>T59[Data]<ETX>[LRC] (request frame)

<STX>T5A[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns stored contactless configuration identifiers.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T59 | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5A | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T59 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T5A. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T5B    Delete Contactless Configuration

Format:        <STX>T5B[Data]<ETX>[LRC] (request frame)

<STX>T5C[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Deletes stored contactless configuration.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5B | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5C | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T5B request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T5C. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T5D    Contactless Housekeeping

Format:        <STX>T5D[Data]<ETX>[LRC] (request frame)

<STX>T5E[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Runs contactless housekeeping compatibility operation.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5D | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5E | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T5D request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T5E. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T5F    Load Contactless DRL Configuration

Format:        <STX>T5F[Data]<ETX>[LRC] (request frame)

<STX>T5G[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores dynamic reader limit configuration.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5F | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5G | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T5F request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T5G. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T5H    Delete Contactless DRL Configuration

Format:        <STX>T5H[Data]<ETX>[LRC] (request frame)

<STX>T5I[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Deletes dynamic reader limit configuration.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5H | 3 | Message ID |
| [Payload] | Variable | Configuration packet or command-specific selector. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T5I | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Reason/Error] | Variable | Optional reason and error message. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T5H request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T5I. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T11    Contact Application Selection

Format:        <STX>T11[Data]<ETX>[LRC] (request frame)

<STX>T12[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts contact application selection only. Multiple applications are shown to the user for touch or keypad selection.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T11 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T12 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T11 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message T12. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T13    Contact Application Selection Alias

Format:        <STX>T13[Data]<ETX>[LRC] (request frame)

<STX>T12[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Alias of contact application selection.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T13 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T12 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T13 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message T12. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T15    Start Contact Transaction

Format:        <STX>T15[Data]<ETX>[LRC] (request frame)

<STX>T16[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Runs contact EMV transaction. Optional PIN scheme selector may be appended.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T15 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T16 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T15 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message T16. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T17    Complete Contact Online Authorization

Format:        <STX>T17[Data]<ETX>[LRC] (request frame)

<STX>T16[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Completes online authorization and second card decision.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T17 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T16 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T17 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T16. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T19    Add Contact Issuer Script

Format:        <STX>T19[Data]<ETX>[LRC] (request frame)

<STX>T20[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores issuer script data for completion.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T19 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T20 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T19 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T20. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T1D    Overwrite Contact Runtime Data

Format:        <STX>T1D[Data]<ETX>[LRC] (request frame)

<STX>T1E[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Overwrites current transaction data objects.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T1D | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T1E | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T1D request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T1E. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T21    Query Contact Transaction Data

Format:        <STX>T21[Data]<ETX>[LRC] (request frame)

<STX>T22[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns requested transaction tags.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T21 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T22 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T21 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T22. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T25    Get Contact Batch Data

Format:        <STX>T25[Data]<ETX>[LRC] (request frame)

<STX>T26[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns next batch data record.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T25 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T26 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T25 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T26. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T27    Get Contact Online Authorization Data

Format:        <STX>T27[Data]<ETX>[LRC] (request frame)

<STX>T28[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns online authorization TLV data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T27 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T28 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T27 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T28. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T29    Get Contact Reversal Data

Format:        <STX>T29[Data]<ETX>[LRC] (request frame)

<STX>T2A[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns reversal TLV data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T29 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T2A | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T29 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T2A. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T2B    Get Contact Online Data Packet

Format:        <STX>T2B[Data]<ETX>[LRC] (request frame)

<STX>T2C[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns online data in single final packet format.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T2B | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T2C | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T2B request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T2C. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T2H    Get Contact Transaction Data Packet

Format:        <STX>T2H[Data]<ETX>[LRC] (request frame)

<STX>T2I[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns online, batch, or reversal data in packet format.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T2H | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T2I | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T2H request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T2I. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T31    Contact Card Data Read

Format:        <STX>T31[Data]<ETX>[LRC] (request frame)

<STX>T12[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts non-financial contact card data read.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T31 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T12 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T31 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message T12. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T33    Fast Contact Read

Format:        <STX>T33[Data]<ETX>[LRC] (request frame)

<STX>T34[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts fast contact data/authorization read and returns packetized result.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T33 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T34 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T33 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message T34. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T35    Non-EMV Contact Transaction

Format:        <STX>T35[Data]<ETX>[LRC] (request frame)

<STX>T16[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Starts non-EMV contact transaction flow.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T35 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T16 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T35 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message T16. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T37    Contact PIN Management

Format:        <STX>T37[Data]<ETX>[LRC] (request frame)

<STX>T38[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Currently returns unsupported failure.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T37 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T38 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T37 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T38. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T3C    Force Complete Non-EMV Contact Flow

Format:        <STX>T3C[Data]<ETX>[LRC] (request frame)

<STX>T3C[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Completes non-EMV card-data flow.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T3C | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T3C | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T3C request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T3C. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T61    Start Contactless Transaction

Format:        <STX>T61[Data]<ETX>[LRC] (request frame)

<STX>T62[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Runs contactless EMV transaction. Optional PIN scheme selector may be appended.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T61 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T62 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T61 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message T62. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T63    Query Contactless Transaction Data

Format:        <STX>T63[Data]<ETX>[LRC] (request frame)

<STX>T64[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns requested contactless tags.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T63 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T64 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T63 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T64. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T65    Query Contactless Online Data

Format:        <STX>T65[Data]<ETX>[LRC] (request frame)

<STX>T66[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns contactless online authorization data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T65 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T66 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T65 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T66. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T67    Query Contactless Payment Scheme

Format:        <STX>T67[Data]<ETX>[LRC] (request frame)

<STX>T68[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Returns selected payment scheme code.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T67 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T68 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T67 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T68. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T71    Complete Contactless Online Authorization

Format:        <STX>T71[Data]<ETX>[LRC] (request frame)

<STX>T62[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Completes contactless online authorization.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T71 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T62 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T71 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T62. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T73    Add Contactless Issuer Script

Format:        <STX>T73[Data]<ETX>[LRC] (request frame)

<STX>T74[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Stores contactless issuer script data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T73 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T74 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T73 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T74. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T75    Load Contactless Revocation List

Format:        <STX>T75[Data]<ETX>[LRC] (request frame)

<STX>T76[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Loads revocation list data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T75 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T76 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T75 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T76. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T77    Load Contactless Exception List

Format:        <STX>T77[Data]<ETX>[LRC] (request frame)

<STX>T78[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Loads exception list data.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T77 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T78 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T77 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Validate command format and process request. If successful, send Message T78. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T81    Start Contactless EMV Transaction

Format:        <STX>T81[Data]<ETX>[LRC] (request frame)

<STX>T62[Data]<ETX>[LRC] (response frame)

Message length:  Variable.

Usage:           Compatibility command to start contactless EMV transaction.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T81 | 3 | Message ID |
| [Payload] | Variable | Transaction request, host response, DOL, or command-specific data. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T62 | 3 | Message ID |
| [Status] | 1 | '0' success, '1' failure. |
| [Result/Data] | Variable | Transaction result code, reversal flag, TLV data, or reason/error fields. |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T81 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Accept valid request and start the operation. Send the response when the operation completes. If successful, send Message T62. If format is invalid, send <EOT> or command-specific failure. |
| Verify response frame. If valid, send <ACK>. If LRC error, send <NAK>. If canceling the session, send <EOT>. | -> |  |

Message T1C    Cancel Contact Transaction

Format:        <STX>T1C[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Cancels active contact transaction.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T1C | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T1C request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message T23    Clear Contact Transaction Log

Format:        <STX>T23[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Clears transaction log state.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T23 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T23 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message T34    Fast Contact Packet Acknowledgement

Format:        <STX>T34[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Host compatibility acknowledgement for fast contact packets.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T34 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T34 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message T38    PIN Management Response Identifier

Format:        <STX>T38[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Compatibility response identifier.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T38 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T38 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message T6C    Cancel Contactless Transaction

Format:        <STX>T6C[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Cancels active contactless transaction.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T6C | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T6C request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

Message T72    Contactless Compatibility Acknowledgement

Format:        <STX>T72[Data]<ETX>[LRC] (request frame)

<ACK>/<NAK>/<EOT> only unless an error frame is specified.

Message length:  Fixed.

Usage:           Accepted no-op used by some hosts.

Message element:

| Request frame (HOST to PINPAD) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <STX> | 1 | <02> |
| T72 | 3 | Message ID |
| <ETX> | 1 | <03> |
| [LRC] | 1 | Checksum |

Message element:

| Response frame (PINPAD to HOST) |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| <ACK> | 1 | Command accepted |
| <NAK> | 1 | Transmission error |
| <EOT> | 1 | Format error, timeout, or canceled session |

Message flow:

| HOST | Dir. | PINPAD |
| --- | --- | --- |
| Message T72 request frame | -> |  |
|  | <- | <ACK>/<NAK>/<EOT> |
|  | <- | Apply command action. No data frame is sent for a normal accepted command. |

# Appendix A    Fixed Prompt Identifiers

Prompt table:

| Data entry prompts |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| 001 | 3 | ACCOUNT NUMBER |
| 003 | 3 | ENTER CUST ID |
| 004 | 3 | ENTER AMOUNT |
| 015 | 3 | ENTER CASH BACK |
| 024 | 3 | ENTER |
| 100 | 3 | ACCOUNT NUMBER |
| 114 | 3 | CUSTOMER REF |
| 115 | 3 | CUSTOMER REF NO. |
| 126 | 3 | ENTER BADGE # |
| 129 | 3 | ENTER CASH BACK |
| 134 | 3 | ENTER CUST REF |
| 200 | 3 | ENTER OTP |
| 201 | 3 | ENTER VERIFICATION CODE |
| 202 | 3 | ENTER CVV2 |
| 203 | 3 | ENTER SMS OTP |
| 204 | 3 | ENTER AUTHENTICATOR CODE |
| 205 | 3 | ENTER TEMPORARY PASSWORD |

Prompt table:

| PIN prompts |  |  |
| --- | --- | --- |
| Field | Length | Value and description |
| 001 | 3 | ENTER PIN |
| 002 | 3 | REENTER PIN |
| 006 | 3 | ENTER NEW PIN |
| 007 | 3 | CONFIRM NEW PIN |
| 024 | 3 | ENTER |